Group34 Zecheng Zuo’s Question

The ARM Cortex-M4 provides multiple addressing modes facilitating efficient access to arrays and complex data structures. Consider a 3D sensor data array defined in C as:

uint16_t sensorData[4][5][6];（25 points）

(a) Design assembly routines to access an arbitrary element sensorData[i][j][k] using:

- PC-relative addressing to load the base address of the array

- Register indirect addressing with scaled offsets to compute the element address

Provide detailed instruction sequences, explaining how each addressing mode is applied and how offsets are calculated.(10 points)

(b) Analyze the trade-offs involved in using PC-relative addressing and register indirect addressing in terms of code size, execution speed, and instruction complexity, especially considering constraints typical in embedded systems.(5 points)

(c) Explain how modern C compilers optimize multi-dimensional array accesses for ARM Cortex-M4 targets, including the role of loop unrolling, register allocation, and instruction scheduling.(10 points)

Answer by Jiaxun Cai:

(a)
Use LDR with PC-relative addressing to load the base address of sensorData into a register (e.g., R0). (2 points)

Then calculate the offset based on indices i, j, and k stored in registers (R1, R2, R3). (2 points)

Multiply i by 30 (5×6), j by 6, add them with k, and shift left by 1 (multiply by 2 bytes per element). Finally use LDRH R7, [R0, R5] to load the 16-bit element.(6 points)

(b)
PC-relative addressing keeps code compact because the literal pool stores the base address nearby, saving memory. (2 points)

Register indirect with scaled offset is flexible but requires extra instructions and registers, impacting speed and register pressure. Overall, this combination balances memory efficiency and runtime performance. (3 points)

(c)
Compilers optimize multi-dimensional array access by converting multiplications with constants into shifts and adds, unrolling inner loops for speed, and keeping indices in registers. (5 points)

They also reorder instructions to hide latencies and minimize memory accesses, resulting in code close to handwritten assembly efficiency. (5 points)

Answer by Junyi Dong:

(a)
Load base address of sensorData using LDR R0, =sensorData (PC-relative). (2 points)

Compute offset with MUL and ADD instructions: offset = ((i * 30) + (j * 6) + k) << 1. (6 points)

Use scaled register addressing mode for load: LDRH R4, [R0, R5]. (2 points)

(b)
PC-relative addressing minimizes the code footprint by using a single instruction to load base addresses. Register indirect addressing provides dynamic access but requires several arithmetic instructions, affecting execution cycles. Using both exploits strengths of ARM instruction set. (5 points)

(c)
The compiler replaces multiplications by constants with shifts and adds, allocates loop counters and base pointers to registers, and uses instruction scheduling to overlap computations and memory loads, improving speed and compactness.

Points: Not very clear, can only get 4 points.

Answer by Zijie Lan:

(a)
First, use PC-relative LDR to get the base pointer. Then calculate the linear index for the 3D coordinates. (2 points)

Multiply i by 30 (rows*columns), j by 6 (columns), add k, and multiply by 2 for byte offset. (6 points)

Finally, LDRH loads the element using register indirect with scaled offset addressing. (2 points)

(b)
PC-relative loads reduce memory usage by storing constants close to code. Register indirect scaled offset addressing simplifies pointer arithmetic but increases instruction count and register usage, potentially slowing execution on register-starved MCUs. (5 points)

(c)
C compilers optimize by unrolling loops, precomputing constant multiplications as shifts, and keeping indices in registers. (4 points)

They also try to reduce load/store overhead by keeping frequently accessed data in registers or cache when available. (6 points)

Answer by Yan Li:

(a)
Use a PC-relative LDR instruction to load the address of the 3D array sensorData into a base register. Compute the offset for indices i, j, k using a sequence of MUL and ADD instructions followed by a LSLS to multiply by element size (2 bytes). Access the element with a halfword load using register indirect addressing.

Points: Not very clear of calculating the address, only obtain 6 points.

(b)
Using PC-relative addressing helps reduce program memory size by leveraging the literal pool. Register indirect addressing with scaled offsets allows for flexible and efficient access to array elements. The trade-off involves more complex instruction sequences and the need for more registers, which may impact performance. (5 points)

(c)
The compiler optimizes multi-dimensional array access by simplifying multiplications to shifts and adds, aggressively using registers to hold indices and base addresses, and applying loop unrolling techniques to reduce branching overhead and increase instruction pipeline efficiency. (10 points)

Answer by Mengtian Yuan:

(a)
Start by loading the base address of sensorData using a PC-relative load (LDR). (2 points)

Calculate the linear index using register indirect addressing: multiply i by 30, add j multiplied by 6, add k, then multiply total offset by 2. (6 points)

Use this offset as the index in a LDRH instruction to retrieve the 16-bit data. (2 points)

(b)
PC-relative addressing is space-efficient but can introduce slight delays due to literal pool access. Register indirect scaled addressing provides runtime flexibility but increases instruction count and register use. The overall method balances code density and speed, vital for embedded systems with limited resources. (5 points)

(c)
Compilers optimize by turning multiplications into shifts, assigning registers to reduce memory accesses, scheduling instructions to maximize throughput, and unrolling loops where beneficial, producing assembly that closely resembles hand-optimized code for array access.

(6 points)