Lab 3 – SysTick Timer Interrupt

For this lab exercise, you will develop a sample application for our STM32F401RE lab board that utilises the internal SysTick timer with interrupt. You will also revisit the use of driver functions for the push button and the LCD display. The objective is to try writing interrupt service routine (ISR) and work out how it would work with the main function(loop).

Important note:

- This lab contributes to 2% of your final grade.

- If you have obtained any assistance from someone or AI services to complete this lab, please complete the declaration form in the end of this lab sheet.

- Any source code(s) that you submit are subject to be checked against Internet sources and other students’ submissions for potential plagiarism.

- This lab is an individual work. When you submit, you also confirm that the submission is entirely your own work and agree to have your work checked again plagiarism.

Useful references / sources for this lab practice

- Lab 3 Keil uVision project (QMPlus)

- Your own source codes from labs 1 and 2

Exercise 1

Download and extract the given project to a folder that contains NO Chinese characters.

Open the project in Keil uVision software

Navigate to design sources on the left pane and open “main.c”.

Study the given function main_sample().

Build the project.

Connect the lab board to your laptop using the USB cable.

Download the image to the lab board. Reset the board.

Describe what do you see in the LCD display.

In the first line, I can see “EBU5477 Lab 3” Second line is “”(An description of a counter that starts from nothing, then goes to the number one, and increments by one every second)

Now, start debugging and set a breakpoint at any C statement inside the callback function systick_isr().

Run until the program pauses at the breakpoint, record the time t1 (left window pane > Internal > Sec).

Run the program again until it pauses, record the time t2 again.

Also, calculate the time difference between t1 and t2 (i.e. t2 - t1).

T1:2.52738348 T2:3.52738298 The time difference between t1 and t2 is 0.99999950

Copy the contents of main_sample() to main1(), and then modify the statements in main1() such that

- the LCD shows the number of minutes (mm), seconds (ss) and 1/100s of a second (xx) in the following format:
mm:ss.xx

You will need to adjust the argument to timer_init() and modify the ISR.

Update main() to run main1() instead of main_sample().

Then test your program by building and downloading to the lab board. If the result is correct, proceed to the next exercise; if not, start a Debug Session to inspect and fix the problem.

Exercise 2

In this exercise, you will create a mini game to record the speed of the player in entering your own name in Pinyin. The rules are as follows:

- A user types your name in Pinyin, all letters in lower cases, via the serial/UART port.

- Timer starts to count just before the processor reads the serial input for the first time.

- If the character matches the name, then it continues; if it does not, timer is reset to 00:00.00 and the user has to start over again.

- If all characters in your name is matched, timer stop and the time spent will be shown in the LCD display in the same format as in exercise 1.

Now, inside main2(), write a program for the mini game. Make sure you initialise and enable the serial port properly.

Change main() to run main2() instead.

Build the project and download the image to the lab board.

Verify your program on the lab board with the PuTTY serial terminal.

In this box, write down your best record with this mini game (mm:ss.xx) .

“Success! Timer: 00:01.38”

You have now completed this lab practice. Well done! If time allows, give the following exercise a go. Don’t forget to get ready your files and submit them to QMPlus.

Exercise 3 (Optional)

Based on the new skills in the previous exercises and labs, choose/design a mini game that also uses the push button with or without serial port.

Implement your game in main3().

In this box, describe briefly your game.

When the player presses the on-board USER button, a five-second round begins. During this window every additional button press scores one point, encouraging the player to tap as fast as possible. A 10 ms SysTick keeps time; once 500 centiseconds (5 s) have elapsed the round ends automatically and the LCD displays “Score: N” with the total taps registered. Pressing the button again resets the game and starts a new round. The entire implementation uses simple polling for the button and a lightweight state machine, requiring no external interrupts or serial port.3 Act fast – The moment you see “PRESS NOW!”, hit the button immediately. 4 See your score – Your reaction time (in ms) pops up, and you can try again to beat it.

Declaration

Please declare any form of assistances from persons or AI services that you used to complete this lab practice. If none, please it empty.

Submission

- Word document of this lab sheet with your answers

- main.c (only) with all functions completed. If you add or modify any other C source files, submit them too.

Please submit the above documents to the corresponding QMPlus assignment pages. The deadline is usually a week after the lab session and will be stated clearly on QMPlus.

- End of Lab Practice -