Mini-Project – Design System Architecture and Create a Prototype of an Embedded System for a Digital Ohmmeter

After the basic training from the lab exercises about the features available on our STM32F401RE lab board, you are now ready to design an embedded system by first creating a system architecture using the use of diagrams (context, block, organigram). You will then demonstrate your design ideas using your programming skills on the lab board and create a simplified prototype of the system.

Your basic task is to design a Digital Ohmmeter, which reads the analogue voltage from a resister network (voltage divider) with one known resister (Rk) and other unknown resistor (Ru). The input analogue voltage will be converted to digital counterpart and using appropriate formulas for voltage divider and ADC you need to calculate value of Ru. The Ohmmeter must read the inputs continuously.

Important note:

- This lab contributes to 7% of your final grade. The Marking Criteria is given in the end of this document.

- The ethics assignment of 5% will be included with the project. The detail questions are give in this document.

- If you have obtained any assistance from someone or AI services to complete this lab, please complete the declaration form in the end of this lab sheet.

- Any source code(s) that you submit are subject to be checked against Internet sources and other students’ submissions for potential plagiarism.

- This is a Group work. When you submit, you also confirm that the submission is entirely your own group work and agree to have your work checked for plagiarism.

- One person from the group can submit.

Useful references / sources for this lab practice

- Slides 02, 03 from teaching block 1

- All your source codes in the lab exercises

- https://en.wikipedia.org/wiki/Voltage_divider

- https://en.wikipedia.org/wiki/Analog-to-digital_converter

- https://en.wikipedia.org/wiki/Ohmmeter

Group Members:

Write the details of your group:

Group ID: 			<Example XX>

Board Number: 		<Example Board XX>

IOT/IST:

Number of group members:

Write the details of your group members, (for every member and fill completely):

Name: Yan Li

QMUL Student ID: 221169209

BUPT Student ID: 2022213446

Student role/contribution in this project:

1. Mathematical Modeling

- Derived Ru calculation formula from voltage divider principles

- Implemented boundary condition handling

2. Technical Documentation

- Authored embedded system justification

- Documented mathematical formulation

Name: Junyi Dong

QMUL Student ID:221169195

BUPT Student ID:2022213445

Student role/contribution in this project:

1. Hardware Setup and Configuration

- Handled physical connection of STM32F401RE lab board.

- Configured ADC module to PA1 pin, verified 5V/GND connections for stable analog input.

2. Demo Video Production and Demonstration

- Recorded 5-min video showcasing prototype operation: LCD startup display, real-time resistance updates, and "Invalid" prompt under faults.

- Demonstrated prototype steps in video, explaining hardware setup troubleshooting methods.

Name: Zecheng Zuo

QMUL Student ID:221169081

BUPT Student ID:2022213434

Student role/contribution in this project:

1.Software Development and Hardware Integration

- Wrote and structured the embedded C code for STM32F401RE using Keil uVision.

- Successfully integrated ADC readings with LCD display, implementing real-time resistance calculations.

- Verified the system functionality through testing and ensured accurate hardware-software coordination.

2. Demonstration Video Recording and Editing

- Planned and recorded a step-by-step demonstration of the digital ohmmeter prototype.

3. Technical Documentation

- Contributed to writing the project worksheet, including system descriptions, mathematical formulation, and architectural diagrams.

- Ensured that documentation was clear, technically accurate, and aligned with project requirements.

Name: Jiaxun Cai

QMUL Student ID:221169346

BUPT Student ID:2022213448

Student role/contribution in this project:

1.Responsible for physical hardware setup and circuit connections.

2.Connected the voltage divider circuit to the STM32F401RE board and ensured proper signal routing to the ADC input pin.

Name:Zijie Lan

QMUL Student ID:221169232

BUPT Student ID: 2022213450

Student role/contribution in this project:

1.Took charge of editing and producing the demonstration video.

2.Recorded real-time testing of the digital ohmmeter and explained key system functions.

Name: 	Mengtian Yuan

QMUL Student ID:221169162

BUPT Student ID: 2022213442

Student role/contribution in this project:

1.Led the documentation process for the project report.

2.Drafted descriptions for the system design, mathematical formulation, and block diagrams.

Step 1: the basic requirements

You can design your embedded system of the Ohmmeter as you like but it should do following basic functions:

- Read the analogue input.

- Convert that input to equivalent useful value using appropriate formulation.

- Display the result on the LCD.

Provide a brief description of your embedded system design as below: (150-200 words) 
(Please type in the box, no handwriting or pictures please)

Our STM32F401RE-based digital ohmmeter implements: 1. Analog Acquisition: PA1 (ADC1) reads voltage from a divider circuit (Rk =2kΩ + Ru) 2. ADC Conversion: 12-bit ADC (0-4095 range) with Vref=3.3V 3. Resistance Calculation: Using Ru= Rk ×VoutVin-Vout where Vin=5.0V 4. LCD Display: Refreshed every 500ms with "Invalid" for fault conditions Operating in a continuous loop: - Sequential execution: ADC sampling → Ru calculation → LCD update - Direct hardware control via GPIO (LCD) and ADC peripherals - Zero external components per project constraints - Real-time performance with bounded latency

Why do you think it is an embedded system? Provide some justification (150-200 words)
(Please type in the box, no handwriting or pictures please)

This system embodies embedded characteristics through: 1. Resource Constraints: Optimized for STM32F401RE's 512KB Flash/96KB RAM 2. Real-Time Operation: Fixed 500ms sampling interval meets timing guarantees 3. Dedicated Function: Single-purpose resistance measurement 4. Hardware Integration: - Bare-metal register access (ADC/GPIO) - No OS abstraction layers - Direct peripheral control 5. Standalone Operation: Autonomous execution post-programming 6. Deterministic Behavior: Time-predictable execution cycles 7. Peripheral-Centric Design: Hardware-defined I/O (ADC pins, LCD interface)

Provide a brief description of your mathematical formulation for your embedded system design as below: (150-2050 words) 
Remember, you will read the analogue voltage from voltage divider via ADC; and need to drive a formula to convert the input voltage to resistive value.

(Please type in the box and use equation editor for mathematical formulas, no handwriting or pictures please)

Basic Concept Our ohmmeter uses a simple circuit with two resistors in series: - Rk (known resistor = 2000Ω) - Ru (unknown resistor we want to measure) We measure the voltage (Vout) across Ru and calculate its value. Step-by-Step Derivation - Voltage Divider Formula The voltage across Ru is: Vout = Vin × RuRk+Ru - Solving for Ru Let's rearrange the formula to find Ru: Vout × (Rk + Ru) =Vin × Ru Vout ×Rk +Vout × Ru = Vin×Ru Vout× Rk = Ru× Vin- Vout Final formula: Ru= Rk ×VoutVin-Vout - Converting ADC Value to Voltage The microcontroller converts voltage to a number (ADC_value) between 0-4095: Vout =ADC_value × 3.3V4095 - Complete Formula Combine both formulas: Ru= ADC_value × 3.3×200040955.0 - ADC_value × 3.34095 Key Parameters Symbol; Meaning; Value. Rk; Known resistor; 2000 Ω. Vin; Input voltage; 5.0 V. Vref; ADC reference voltage; 3.3 V. ADC_value; ADC reading; 0-4095. Special Cases - Short Circuit (ADC_value = 0): Ru = 0 Ω - Open Circuit (ADC_value ≥ 4095): Display "Invalid" - Calculation Error (Vin ≤ Vout): Display "Invalid"

Step 2: Context Diagram and Block Diagram

Based your topic, draw a context diagram to provide as much details as you can about the system. Think of interactions from users and/or other systems. Examples can be found in the slides.

Context diagram +--------------------------+ +----------------+ +-----------------+ ¦ User Interaction ¦ ¦ External System ¦ ¦ Physical World ¦ ¦ - Input via Lab Board Controls ¦<--->¦ - 5V Power Supply ¦<----->¦ - Voltage Divider ¦ ¦ - Read LCD Display ¦ ¦ - Ground (GND) ¦ ¦ - Unknown Resistor ¦ +--------------------------+ +-----------------+ +-----------------+ ^ ^ ^ ¦ ¦ ¦ v v v +----------------------------+ +-----------------------+ +-----------------------+ ¦ STM32F401RE Microcontroller ¦ ¦ ADC Conversion ¦ ¦ LCD Display ¦ ¦- GPIO Control (LCD Interface) ¦<--->¦ - 12-bit ADC (PA1 Pin) ¦<--->¦ - Displays Resistance Value ¦ ¦ - ADC Configuration (PA1) ¦ ¦ - Converts Analog to Digital ¦ ¦ - Shows "Invalid" on Faults ¦ ¦ - Resistance Calculation Algorithm ¦ ¦ - 0-4095 Range (3.3V Ref) ¦ ¦ ¦ +----------------------------+ +------------------------+ +------------------------+

Then identify the physical objects and hardware components that you need. Represent them as a block in the following block diagram. Annotate the connections between the related blocks with information like the hardware/software interfaces, electrical connections etc.

Block Diagram +-------------------------+ +----------------------+ +------------------------+ ¦ Power Supply ¦ ¦ STM32F401RE ¦ ¦ LCD Display ¦ ¦ - 5V Pin (V_in = 5.0V) ¦<--->¦ - ADC1 (12-bit, PA1) ¦<--->¦ - GPIO-Controlled Interface ¦ ¦ - GND Pin (Common Ground) ¦ ¦ - GPIO Pins (LCD Control) ¦ ¦ - 500ms Refresh Cycle ¦ +-------------------------+ +----------------------+ +-----------------------+ ^ ¦ v +-----------------------+ +-----------------------+ ¦ Analog Input ¦ ¦ Resistance Calculation ¦ ¦ - PA1 Pin (V_out Sampling)¦ ¦ - Voltage Divider Formula ¦ ¦ - Equivalent to Rk (2kΩ) ¦<--->¦ - ADC Value to Voltage ¦ ¦ and Ru Voltage Divider ¦ ¦ - Fault Detection (Invalid) ¦ +-----------------------+ +-----------------------+

Make the schematic/Circuit diagram if required by your design. If not required, explain why.

Schematic/Circuit Diagram Rk=2KΩ Ru GND Vin=5v Vout

Step 3: design and program a prototype

Make use of the lab board, identify the devices that you are going to use for your system. Please note that you are not allowed to add any electronic components to the lab board. As this is only a prototype, you may use appropriate replacement, for instance, reading the analogue input form battery or from the switch (as you have done in lab experiments). You can explain the voltage divider formulation for converting the analogue input voltage to appropriate resistance value. In the following box, provide a list of devices and the actual representations.

Device representation

(Please type in the box, no handwriting or pictures please)

- Device: LCD Display Representation: Utilize the LCD connected to the STM32F401RE board to present measurement - related information. Description: At the start of the program, it shows the message "Measure Resistor". Subsequently, it continuously updates and displays the calculated resistance value. If the calculated resistance is less than 1.0 Ohm, it shows the message "Invalid". Otherwise, it shows the resistance value in the format "R=X.X Ohm", where X.X is the calculated resistance value with one - decimal - place precision. The display is refreshed every 500 milliseconds to provide real - time measurement results. - Device: STM32F401RE Microcontroller Representation: Acts as the central processing unit of the digital ohmmeter system, integrating various functions such as ADC conversion, resistance calculation, and LCD control. Description: Initializes the ADC on pin PA1 to sample the analog voltage. Performs 12 - bit ADC conversion on the input voltage, and uses the converted digital value to calculate the unknown resistance based on the voltage - divider principle. Controls the LCD display through GPIO pins, sending data and commands to update the display content. Manages the overall operation cycle, ensuring that ADC sampling, resistance calculation, and LCD update are executed sequentially every 500 milliseconds. - Device: 12 - bit ADC (Integrated in STM32F401RE) Representation: The built - in 12 - bit ADC module in the STM32F401RE, connected to pin PA1 to sample the analog voltage. Description: Receives the analog voltage from the equivalent voltage - divider circuit through pin PA1. Converts this analog voltage into a 12 - bit digital value within the range of 0 - 4095, with a reference voltage of 3.3V. The converted digital value is then used by the microcontroller for subsequent resistance calculations. - Device: Voltage Input Source Representation: The 5V pin on the STM32F401RE lab board, serving as the input voltage for the theoretical voltage - divider circuit. Description: Represents the input voltage Vin (set to 5.0V) in the voltage - divider formula used for resistance calculation. It provides the reference voltage for the voltage - divider circuit, and the voltage difference between it and the voltage at pin PA1 is used to calculate the unknown resistance. - Device: Ground (GND) Representation: The GND pin on the STM32F401RE lab board, providing a common ground reference for the entire circuit. Description: Serves as the zero - potential reference point for the analog input, ADC conversion, and LCD display. Ensures the normal operation of the circuit by providing a stable ground connection.

Once this is done, you can start programming the system using the techniques you learned from the lab exercises.

Please make sure that we will try to reproduce your results when we mark and reasonable comments (by yourself) to explain your code are recommended using comments in your code.

Type X in the following box to confirm that your code is developed using Keil uVision v 5.36 with our standard lab board with LCD shield. You have to submit the whole Keil project as a zip to QMPlus.

X

Step 4: Organigram and demo video

Finally document your source codes by summarising it using an organigram (see slides for examples). This diagram should show carefully the relationships between different functions (including the driver functions) in your program. Remember to indicate the sharing of software or hardware resources.

Organigram +-------------------+ ¦ main() ¦ ¦------------------- ¦ ¦ Initialize peripherals, ¦ ¦ loop to read ADC, ¦ ¦ calculate resistance, ¦ ¦ and display on LCD ¦ +-------------------+ ¦ +---------------+-----------------------+ ¦ ¦ ¦ +------------ + +-----------------+ +----------------+ ¦ ADC_Init() ¦ ¦ ADC_Read() ¦ ¦ LCD_Display() ¦ ¦-------------¦ ¦------------------¦ ¦-----------------¦ ¦Configure GPIO ¦ ¦ Start ADC conversion¦ ¦ Format resistance ¦ ¦ for analog ¦ ¦ Wait for EOC flag ¦ ¦ to string and call ¦ ¦ input, enable ¦ ¦ Read digital value ¦ ¦ LCD_Print() to show ¦ ¦ ADC1 ¦ ¦ from ADC data reg ¦ ¦ result on LCD screen¦ +------------+ +-------------------+ +----------------+ ¦ +------------------------+ ¦ Calculate_Resistance() ¦ ¦------------------------ ¦ ¦ Convert ADC value to voltage ¦ ¦ Apply voltage divider formula ¦ ¦ to estimate Ru based on Rk ¦ +------------------------+

Lastly, record a 5-minute demonstration video to

(i) show the working functions of your prototype, and

(ii) explain 1-2 key knowledge/skills/ideas that you’ve learned through this mini-project.

Type X in the following box to confirm that you have recorded the demo video on your own and submit the video on QMPlus.

X

Step 5: Ethical Questions related to project

For the Ethical part of the project answer the following questions

How can we ensure that the Digital Ohmmeter provides accurate and reliable resistance readings, and what are the ethical implications if incorrect data is used in real-world decision-making or safety-critical applications? (150-250 words)

To ensure a Digital Ohmmeter provides accurate and reliable resistance readings, several measures should be taken. First, calibrate the device regularly using known reference resistors to correct for any drift in its internal circuitry. Second, verify the test leads for conductivity and minimal resistance, as worn or loose leads can introduce errors. Third, avoid measuring live circuits to prevent damage to the meter and inaccurate readings. Fourth, select the appropriate range on the ohmmeter to ensure the measurement falls within its most precise interval. Finally, store the meter in a stable environment free from extreme temperatures or humidity, which can affect its internal components. If incorrect resistance data from a faulty ohmmeter is used in real-world applications, especially safety-critical contexts like electrical wiring or industrial equipment, the ethical implications are severe. Inaccurate readings could lead to faulty system designs, unexpected equipment failures, or even safety hazards such as electrical shocks or fires. This endangers human lives and violates the ethical responsibility to prioritize public safety. Additionally, using flawed data in decision-making undermines professional integrity and trust in technical expertise, potentially causing financial losses and reputational damage. Ensuring measurement accuracy is not just a technical requirement but an ethical obligation to uphold reliability and accountability in engineering and scientific practices.

To what extent are engineers ethically responsible for the accuracy and consequences of the embedded systems they design; especially when those systems interact with the physical world? (150-250 words)

Engineers bear profound ethical responsibility for the embedded systems they design, especially when these systems integrate with critical sectors like automotive, medical, or industrial domains. A minor software glitch or hardware flaw in such contexts can escalate into catastrophic accidents, physical injuries, or even loss of life—highlighting the direct correlation between design integrity and public safety. While many projects involve collaborative teams, individual engineers cannot abdicate responsibility by invoking "division of labor." Each team member has a moral obligation to flag potential risks, even if issues arise outside their designated scope. This duty extends beyond technical compliance: rigorous testing, adherence to safety standards, and proactive consideration of user interactions (including edge cases or unintentional misuse) are non-negotiable. Cost-cutting measures or deadline pressures often tempt teams to compromise on thoroughness, but such shortcuts inherently violate ethical mandates. Engineers face not only legal repercussions for negligent design but also moral accountability for foreseeable harm.

Declaration

Please declare any form of assistances from persons or AI services that you used to complete this mini-project. If none, please write “NONE”.

Submission

- Word document of this sheet with your answers
Type your answers (no handwriting please) in boxes provided.

- Do not attach pictures (other than diagram).

- The whole Keil project with all source codes as a zip, 
as explained in Step 3: design and program a prototype.

- Demonstration video (5 minutes), 
as explained in Step 4: Organigram and demo video

Please submit the above to the corresponding QMPlus assignment pages. The deadline will be stated clearly on QMPlus.

Marking Criteria

System Architecture (context diagram, mathematical formula derivation, block diagram, organigram) (40%)

- Clarity

- Amount of details

- Cohesion with actual implementation of prototype

Prototype design and source code (30%)

- Appropriate use of devices

- Proper program design structure (use of memory, interrupts, functions)

- Clear documentations with comments, professional format (indentation, names of variables)

Demonstration Video (30%)

- Achievement of the prototype (board demonstration)

- Reflection (knowledge and skills learned)

- End of Mini Project -